SIMATS ENGINEERING

Department of Computer Science & Engineering

ASSESSMENT TOOL 1- EXPERIMENTS

| Course Code: | CSA1205 | Course Title: | Computer Architecture for Multicore Systems |
| --- | --- | --- | --- |
| CO Assessed: | CO3 | Assessment: | ASSESSMENT TOOL 1- EXPERIMENTS |
| Weightage: | 40% | Total Marks: | 25 |
| CO3 Statement: | Design processor organization by constructing pipelined datapath structures, identifying hazard types (data, control, structural), and comparing superscalar techniques such as out-of-order execution, speculative execution, and simultaneous multithreading (SMT). (BL6) |  |  |
| Student Name: |  | Reg.No.: |  |
| Department: |  | Date: |  |

ANNEXURE A

- Design and simulate a system that performs register transfer operations along with basic arithmetic and logic operations such as addition, subtraction, AND, and OR. Use multiple registers to demonstrate how data is transferred between them during execution, and analyze the sequence of operations and the role of the ALU in processing data.

- Create a model of a computer system using single-bus and multi-bus organization, and perform data transfer operations between registers, memory, and I/O. Compare the time taken and efficiency of both approaches, and analyze how multiple bus structures improve parallel data transfer and reduce bottlenecks.

- Simulate a 5-stage instruction pipeline (Fetch, Decode, Execute, Memory, Write-back) and execute a set of instructions. Measure the performance in terms of speedup and throughput, and compare it with a non-pipelined system to evaluate the effectiveness of pipelining.

- Develop a pipeline execution scenario where data hazards, control hazards, and structural hazards occur. Observe their impact on instruction execution, and implement techniques such as stalling, data forwarding, and branch prediction to resolve these hazards and improve performance.

- Analyze a processor supporting superscalar execution, including out-of-order and speculative execution, and extend the study to include hyper-threading and simultaneous multithreading (SMT). Evaluate how multiple instructions are executed in parallel and compare the performance improvements in terms of instruction throughput and resource utilization.

Code of Conduct:

I ____________________________________________ (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

MARKS ALLOCATION

|  | Understanding of Concepts (20%) | Experimental Design (20%) | Use of Tools (25%) | Analysis of Results (20%) | Documentation (15%) |
| --- | --- | --- | --- | --- | --- |
| Q1 |  |  |  |  |  |
| Q2 |  |  |  |  |  |
| Q3 |  |  |  |  |  |
| Q4 |  |  |  |  |  |
| Q5 |  |  |  |  |  |

RUBRICS FOR CALCULATION:

| Criteria | Weightage | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Understanding of Concepts | 25 | Demonstrates thorough understanding and effectively integrates multiple concepts/technologies | Good understanding with proper integration of most concepts | Basic understanding; limited or partial integration | Poor understanding; unable to integrate concepts |
| Experimental Design | 30 | Well-planned, systematic implementation with optimal solution | Correct implementation with minor issues | Basic implementation with errors or inefficiencies | Incorrect or incomplete implementation |
| Use of Tools | 20 | Effective and advanced use of tools/technologies with proper configuration | Appropriate use of tools with correct execution | Limited or inconsistent use of tools | Incorrect or minimal use of tools |
| Analysis of Results | 15 | Insightful analysis with accurate interpretation and validation of results | Good analysis with reasonable interpretation | Basic analysis with limited insights | Poor or incorrect analysis of results |
| Documentation | 10 | Clear, well-structured documentation with diagrams, code, and results | Organized documentation with necessary details | Basic documentation with missing details | Poor or incomplete documentation |

1. Aim

To design and simulate a processor datapath that performs register transfer operations along with basic arithmetic and logic operations such as addition, subtraction, AND, and OR, using multiple registers, and to analyze the role of the Arithmetic Logic Unit (ALU) during execution. This directly addresses Experiment 1 in the assessment.

2. Objectives

- To understand register-to-register data transfer.

- To design a simple processor datapath using multiple registers.

- To perform arithmetic operations using an ALU.

- To perform logical operations using an ALU.

- To understand how control signals coordinate register transfers.

- To analyze the sequence of operations during instruction execution.

3. Concept

A register is a small, high-speed storage location inside the processor. During instruction execution, data is transferred between registers and processed by the ALU.

For this experiment, consider four registers:

- R1 – Source Register 1

- R2 – Source Register 2

- R3 – Destination/Temporary Register

- R4 – Result Register

The ALU performs:

| Operation | Description |
| --- | --- |
| ADD | R1 + R2 |
| SUB | R1 − R2 |
| AND | R1 AND R2 |
| OR | R1 OR R2 |

4. Proposed Datapath

┌──────────┐

R1 ────►│          │

│   ALU    │────► R4

R2 ────►│          │

└────┬─────┘

│

Control Unit

│

┌─────────┴─────────┐

│ Register Control   │

└────────────────────┘

The Control Unit selects the required ALU operation and controls which register supplies the input and receives the output.

5. Register Transfer Operations

Register transfer can be represented using Register Transfer Language (RTL).

Examples:

R3 ← R1

R4 ← R2

R3 ← R1 + R2

R4 ← R1 − R2

R3 ← R1 AND R2

R4 ← R1 OR R2

Here, the symbol ← indicates that the contents on the right side are transferred to the register on the left side.

6. Experimental Design

Assume:

R1 = 1010

R2 = 0011

Therefore:

- R1 = 10 in decimal

- R2 = 3 in decimal

The ALU receives R1 and R2 as inputs and performs the selected operation.

7. Operation 1 – Register Transfer

Instruction:

R3 ← R1

Initial condition:

R1 = 1010

R3 = 0000

After execution:

R3 = 1010

Thus, the contents of R1 are successfully transferred to R3.

8. Operation 2 – Addition

Instruction:

R4 ← R1 + R2

Calculation:

1010

+ 0011

------

1101

Therefore:

R4 = 1101

Decimal result:

10 + 3 = 13

9. Operation 3 – Subtraction

Instruction:

R4 ← R1 − R2

Calculation:

10 − 3 = 7

Binary result:

R4 = 0111

10. Operation 4 – AND

Instruction:

R4 ← R1 AND R2

1010

AND 0011

--------

0010

Therefore:

R4 = 0010

11. Operation 5 – OR

Instruction:

R4 ← R1 OR R2

1010

OR  0011

--------

1011

Therefore:

R4 = 1011

12. Observation Table

| Step | Operation | R1 | R2 | Result |
| --- | --- | --- | --- | --- |
| 1 | R3 ← R1 | 1010 | 0011 | R3 = 1010 |
| 2 | ADD | 1010 | 0011 | R4 = 1101 |
| 3 | SUB | 1010 | 0011 | R4 = 0111 |
| 4 | AND | 1010 | 0011 | R4 = 0010 |
| 5 | OR | 1010 | 0011 | R4 = 1011 |

13. ALU Role

The ALU is the computational unit of the datapath. It receives operands from registers and performs the operation specified by the control unit.

For example:

R1 ─────┐

│

▼

┌─────┐

R2 ──►│ ALU │───► Result Register

└─────┘

▲

│

Operation Select

The control unit determines whether the ALU performs:

ADD → 00

SUB → 01

AND → 10

OR  → 11

The exact control encoding may vary depending on the simulator implementation.

14. Simulation Procedure

- Create four registers: R1, R2, R3 and R4.

- Initialize R1 and R2 with binary values.

- Connect R1 and R2 to the ALU input.

- Connect the ALU output to the destination register.

- Add control signals for selecting ALU operations.

- Perform a register-transfer operation.

- Execute ADD, SUB, AND and OR operations.

- Record the output after each operation.

- Compare the simulated output with the expected mathematical result.

- Analyze the datapath and ALU operation.

15. Pseudocode

START

Initialize R1 = 1010

Initialize R2 = 0011

R3 ← R1

R4 ← R1 + R2

Display R4

R4 ← R1 − R2

Display R4

R4 ← R1 AND R2

Display R4

R4 ← R1 OR R2

Display R4

STOP

16. Expected Results

The simulation should produce:

Register Transfer → 1010

Addition          → 1101

Subtraction       → 0111

AND               → 0010

OR                → 1011

The results demonstrate that data can be transferred between registers and that the ALU correctly performs both arithmetic and logical operations.

17. Analysis

The experiment demonstrates that processor operations are performed through coordinated interaction between registers, the ALU and the control unit.

The register-transfer operation does not require complex computation; it simply moves data from one register to another. In contrast, arithmetic and logical operations require the ALU to process two operands.

The sequence can be summarized as:

Register Input

↓

Operand Selection

↓

ALU

↓

Operation Result

↓

Destination Register

Using multiple registers allows temporary data and intermediate results to be stored without repeatedly accessing main memory. This contributes to faster processor operation.

18. Result

The designed datapath successfully demonstrated register transfer, addition, subtraction, AND and OR operations. The ALU correctly processed the operands supplied by the registers, while the control mechanism selected the required operation and destination register.

19. Conclusion

The experiment successfully demonstrates the fundamental organization of a processor datapath. Multiple registers provide fast storage for operands and results, while the ALU performs arithmetic and logical operations. The experiment also shows that register transfer operations and ALU operations work together under the control of the processor's control unit, forming the foundation for more advanced processor organizations and pipelined datapaths.

1. Aim

To create a model of a computer system using single-bus and multi-bus organization, perform data transfer operations between registers, memory and I/O, and compare both approaches in terms of transfer time, efficiency, parallelism and bottlenecks. This corresponds directly to Experiment 2 in the assessment.

2. Objectives

- To understand single-bus organization.

- To understand multi-bus organization.

- To simulate data transfers between registers, memory and I/O.

- To compare the number of transfer steps required.

- To analyze the effect of bus structure on processor performance.

- To understand how multiple buses enable parallel data movement.

3. Introduction

A bus is a communication pathway used to transfer data between different components of a computer system.

A processor generally needs to transfer information between:

- Registers

- ALU

- Main memory

- Input/Output devices

Two common organizations are:

- Single-bus organization

- Multi-bus organization

The major difference is the number of independent data paths available for transferring information.

4. Single-Bus Organization

In a single-bus system, all major components share one common bus.

Block Diagram

┌──────────┐

│ Register │

└────┬─────┘

│

▼

═══════════════════

SINGLE BUS

═══════════════════

│ │ │

▼ ▼ ▼

┌─────┐ ┌─────┐ ┌─────┐

│ ALU │ │ MEM │ │ I/O │

└─────┘ └─────┘ └─────┘

Since all components use the same bus, generally only one source can place data onto the bus at a time.

Example

Suppose:

R1 = 25

R2 = 15

To perform:

R3 ← R1 + R2

The processor needs to transfer the operands to the ALU through the common bus.

A simplified sequence is:

Step 1: R1 → Bus

Step 2: Bus → ALU input

Step 3: R2 → Bus

Step 4: Bus → ALU input

Step 5: ALU performs addition

Step 6: ALU result → Bus

Step 7: Bus → R3

Thus, several sequential transfers are required.

5. Multi-Bus Organization

A multi-bus system uses two or more independent buses.

A common example is a three-bus organization containing:

- Bus A

- Bus B

- Bus C

Block Diagram

┌───────────┐

│ Registers │

└─────┬─────┘

│ │

▼ ▼

┌───────┐ ┌───────┐

│ Bus A │ │ Bus B │

└───┬───┘ └───┬───┘

│ │

└────┬────┘

▼

┌─────┐

│ ALU │

└──┬──┘

│

▼

┌───────┐

│ Bus C │

└───┬───┘

│

┌──────┼──────┐

▼ ▼ ▼

R3 MEM I/O

Here, two operands can be supplied to the ALU through separate buses.

For example:

R1 → Bus A ──┐

├──► ALU ──► Bus C → R3

R2 → Bus B ──┘

This significantly reduces the number of sequential transfers.

6. Experimental Design

The same operation is performed using both architectures:

R3 ← R1 + R2

Initial values:

R1 = 25

R2 = 15

Expected result:

R3 = 40

7. Single-Bus Implementation

Transfer Sequence

| CycleOperation |
| --- |
| 1 | R1 → Bus |
| 2 | Bus → ALU input A |
| 3 | R2 → Bus |
| 4 | Bus → ALU input B |
| 5 | ALU performs ADD |
| 6 | ALU → Bus |
| 7 | Bus → R3 |

Total simplified transfer cycles:

7 cycles

The exact number can vary depending on the specific datapath and control implementation.

8. Multi-Bus Implementation

Using two input buses and one output bus:

R1 → Bus A

R2 → Bus B

Both operands can be supplied to the ALU during the same transfer phase.

Transfer Sequence

| CycleOperation |
| --- |
| 1 | R1 → Bus A and R2 → Bus B |
| 2 | ALU performs ADD |
| 3 | ALU result → Bus C → R3 |

Total simplified cycles:

3 cycles

Again, the exact count depends on the simulator/datapath design.

9. Comparison

| FeatureSingle-BusMulti-Bus |
| --- |
| Number of buses | One | Two or more |
| Data transfer | Mostly sequential | More parallel |
| Hardware complexity | Low | Higher |
| Cost | Lower | Higher |
| Transfer time | Higher | Lower |
| Parallel operations | Limited | Better |
| Bus bottleneck | High | Reduced |
| Control complexity | Lower | Higher |
| Performance | Lower | Higher |
| Resource utilization | Limited | Improved |

10. Performance Analysis

Using the simplified experimental cycle counts:

Single-bus = 7 cycles

Multi-bus = 3 cycles

Cycle Reduction

Reduction = 7 − 3

= 4 cycles

Percentage Reduction

Reduction %=77−3​×100 =57.14%

Therefore, under this simplified model, the multi-bus organization reduces the required transfer cycles by approximately 57.14%.

Relative Speedup

Speedup=37​ ≈2.33

Thus, the simplified multi-bus implementation can achieve approximately 2.33× the transfer performance of the single-bus implementation for this particular operation.

These values are experimental-model values based on the transfer sequences above; actual hardware performance depends on the particular datapath, bus widths, control logic and memory timing.

11. Memory Transfer

Consider:

R1 ← Memory[1000]

Single-Bus

Memory → Bus → R1

The shared bus must be controlled so that memory places the required data on the bus and R1 receives it.

Multi-Bus

A separate pathway can be available for memory communication while another bus handles other internal transfers.

This allows the architecture to reduce contention when multiple components need to communicate.

12. I/O Transfer

Consider:

R2 ← Input Device

Single-Bus

I/O → Bus → R2

If another component simultaneously requires the bus, the transfer must wait.

Multi-Bus

A dedicated or additional communication path can allow I/O activity to occur with less interference from other internal transfers.

13. Simulation Procedure

- Create registers R1, R2 and R3.

- Create memory and I/O modules.

- Design the single-bus datapath.

- Connect registers, ALU, memory and I/O to the common bus.

- Initialize R1 and R2.

- Perform R3 ← R1 + R2.

- Record the number of cycles.

- Create a multi-bus datapath.

- Provide separate input buses for the operands.

- Repeat the same operation.

- Record the transfer cycles.

- Compare the results and performance.

14. Pseudocode

START

Initialize R1 = 25

Initialize R2 = 15

// Single-Bus

Transfer R1 to Bus

Transfer Bus to ALU input A

Transfer R2 to Bus

Transfer Bus to ALU input B

Perform ADD

Transfer ALU result to Bus

Transfer Bus to R3

Record single_bus_cycles

// Multi-Bus

Transfer R1 to Bus A

Transfer R2 to Bus B

Perform ADD

Transfer ALU result to Bus C

Transfer Bus C to R3

Record multi_bus_cycles

Compare performance

STOP

15. Observation Table

| ParameterSingle-BusMulti-Bus |
| --- |
| R1 | 25 | 25 |
| R2 | 15 | 15 |
| Expected R3 | 40 | 40 |
| Simplified transfer cycles | 7 | 3 |
| Parallel operand transfer | No | Yes |
| Bus contention | Higher | Lower |
| Relative speedup | 1× | ≈2.33× |

16. Analysis of Results

The experiment demonstrates that both architectures can produce the same computational result:

25 + 15 = 40

However, the method of transferring the operands differs significantly.

In the single-bus architecture, the common bus becomes a shared resource. Multiple transfers have to occur sequentially, creating a potential bottleneck.

In the multi-bus architecture, separate buses allow multiple data transfers to occur during the same transfer phase. This improves parallelism and reduces the number of sequential operations.

The major trade-off is that multi-bus organization requires additional hardware, more complex control logic and greater implementation cost.

Therefore:

More buses

↓

More parallel transfers

↓

Less bus contention

↓

Reduced transfer time

↓

Higher potential performance

17. Advantages

Single-Bus

- Simple architecture

- Lower hardware requirement

- Lower implementation cost

- Easier control design

Multi-Bus

- Faster data transfers

- Greater parallelism

- Reduced bottlenecks

- Better utilization of processor resources

- Improved datapath performance

18. Limitations

Single-Bus

- Bus contention

- Sequential transfers

- Lower potential throughput

- Can become a performance bottleneck

Multi-Bus

- Increased hardware complexity

- Higher cost

- More complicated control signals

- Greater design complexity

19. Result

The single-bus and multi-bus computer organizations were successfully modeled and compared. Both systems performed the required register, memory and I/O data transfers correctly. The multi-bus architecture demonstrated better parallel data-transfer capability and required fewer sequential transfer steps for the selected ALU operation.

20. Conclusion

The experiment demonstrates the importance of bus organization in processor performance. A single-bus architecture provides a simple and economical design but suffers from shared-resource bottlenecks. A multi-bus architecture uses additional communication paths to allow greater parallelism and reduce transfer delays.

Therefore, multi-bus organization provides better potential performance and resource utilization, while single-bus organization remains attractive when simplicity, lower hardware complexity and cost are more important.

This addresses the assessment's requirement to compare transfer time and efficiency and to analyze how multiple buses improve parallel data transfer and reduce bottlenecks.

1. Aim

To simulate a 5-stage instruction pipeline consisting of Fetch, Decode, Execute, Memory, and Write-back, execute a set of instructions, measure performance in terms of speedup and throughput, and compare the pipelined processor with a non-pipelined processor. This directly corresponds to Experiment 3 in the assessment.

2. Objectives

- To understand instruction pipelining.

- To implement the five stages of a processor pipeline.

- To execute multiple instructions concurrently.

- To calculate pipeline execution time.

- To calculate speedup and throughput.

- To compare pipelined and non-pipelined execution.

- To analyze the effectiveness of pipelining.

3. Introduction

Instruction pipelining is a processor design technique in which instruction execution is divided into multiple stages.

Instead of completing one instruction entirely before starting another, different instructions occupy different stages simultaneously.

The five stages used in this experiment are:

- IF – Instruction Fetch

- ID – Instruction Decode

- EX – Execute

- MEM – Memory Access

- WB – Write-back

The basic concept is:

Instruction

↓

┌──────┐

│ IF │ Fetch

└──┬───┘

↓

┌──────┐

│ ID │ Decode

└──┬───┘

↓

┌──────┐

│ EX │ Execute

└──┬───┘

↓

┌──────┐

│ MEM │ Memory

└──┬───┘

↓

┌──────┐

│ WB │ Write-back

└──────┘

4. Five Pipeline Stages

Stage 1 – Instruction Fetch (IF)

The processor obtains the instruction from memory using the Program Counter (PC).

PC → Instruction Memory → Instruction

The PC is then updated to point to the next instruction.

Stage 2 – Instruction Decode (ID)

The fetched instruction is decoded.

The processor identifies:

- Operation

- Source registers

- Destination register

- Immediate values

- Required control signals

Example:

ADD R3, R1, R2

The processor identifies:

Operation = ADD

Source 1 = R1

Source 2 = R2

Destination = R3

Stage 3 – Execute (EX)

The required operation is performed.

For an arithmetic instruction, the ALU performs the calculation.

Example:

R3 = R1 + R2

Stage 4 – Memory Access (MEM)

This stage is mainly required for memory instructions.

For example:

LOAD → Read memory

STORE → Write memory

For an arithmetic instruction such as ADD, this stage may simply pass the result forward.

Stage 5 – Write-back (WB)

The final result is written into the destination register.

Example:

R3 ← ALU Result

5. Instructions Used for Simulation

Consider the following five independent instructions:

I1: ADD R1, R2, R3

I2: SUB R4, R5, R6

I3: AND R7, R8, R9

I4: OR R10, R11, R12

I5: ADD R13, R14, R15

These instructions are chosen to avoid data dependencies so that the basic effect of pipelining can be clearly observed.

6. Non-Pipelined Execution

In a non-pipelined processor, each instruction completes all five stages before the next instruction begins.

I1 → IF → ID → EX → MEM → WB

I2 → IF → ID → EX → MEM → WB

I3 → IF → ID → EX → MEM → WB

I4 → IF → ID → EX → MEM → WB

I5 → IF → ID → EX → MEM → WB

For five instructions:

Tnon−pipeline​=5×5=25 cycles

Therefore:

Non-pipelined execution time = 25 cycles

7. Pipelined Execution

In a pipeline, a new instruction can enter the first stage during every clock cycle.

Pipeline Schedule

| CycleI1I2I3I4I5 |
| --- |
| 1 | IF |  |  |  |  |
| 2 | ID | IF |  |  |  |
| 3 | EX | ID | IF |  |  |
| 4 | MEM | EX | ID | IF |  |
| 5 | WB | MEM | EX | ID | IF |
| 6 |  | WB | MEM | EX | ID |
| 7 |  |  | WB | MEM | EX |
| 8 |  |  |  | WB | MEM |
| 9 |  |  |  |  | WB |

All five instructions are completed after 9 cycles.

8. Pipeline Timing Diagram

Cycle →   1    2    3    4    5    6    7    8    9

I1 IF ID EX MEM WB

I2 IF ID EX MEM WB

I3 IF ID EX MEM WB

I4 IF ID EX MEM WB

I5 IF ID EX MEM WB

This diagram clearly shows how multiple instructions are processed simultaneously.

For example, during cycle 4:

I1 → MEM

I2 → EX

I3 → ID

I4 → IF

Four instructions are simultaneously occupying different pipeline stages.

9. Pipeline Execution Time

For a pipeline containing:

- k stages

- n instructions

and assuming each stage takes one clock cycle:

Tpipeline​=k+(n−1)

For:

k=5,n=5

Therefore:

Tpipeline​=5+(5−1) =9 cycles

10. Speedup Calculation

Speedup is calculated as:

Speedup=Tpipeline​Tnon−pipeline​​

Therefore:

Speedup=925​ Speedup≈2.78​

Thus, the five-instruction example provides approximately 2.78× speedup under the idealized assumptions.

11. Theoretical Maximum Speedup

For a large number of instructions, the ideal pipeline approaches a speedup equal to the number of stages.

For a 5-stage pipeline:

Maximum ideal speedup≈5

However, the actual speedup is usually lower because of:

- Pipeline startup time

- Data hazards

- Control hazards

- Structural hazards

- Unequal stage delays

- Pipeline stalls

12. Throughput

Throughput represents the number of instructions completed per unit of time.

For the simplified simulation:

5 instructions / 9 cycles

Therefore:

Throughput=95​ ≈0.556

So the average throughput is approximately:

0.556 instructions/cycle​

Once the pipeline is full, ideally one instruction can complete every clock cycle.

13. Comparison

| ParameterNon-PipelinedPipelined |
| --- |
| Number of stages | 5 | 5 |
| Instructions | 5 | 5 |
| Total cycles | 25 | 9 |
| Speedup | 1× | 2.78× |
| Parallel execution | No | Yes |
| Instruction overlap | No | Yes |
| Throughput | Lower | Higher |
| Hardware complexity | Lower | Higher |

14. Experimental Procedure

- Define the five pipeline stages.

- Create the instruction set.

- Implement the IF stage.

- Implement the ID stage.

- Implement the EX stage.

- Implement the MEM stage.

- Implement the WB stage.

- Execute the instructions without pipelining.

- Record the number of cycles.

- Execute the same instructions using the pipeline.

- Record the pipeline timing.

- Calculate speedup.

- Calculate throughput.

- Compare the two architectures.

15. Pseudocode

START

Define pipeline stages:

IF, ID, EX, MEM, WB

Define instructions:

I1 = ADD

I2 = SUB

I3 = AND

I4 = OR

I5 = ADD

// Non-pipelined execution

FOR each instruction

Perform IF

Perform ID

Perform EX

Perform MEM

Perform WB

END FOR

Calculate non_pipeline_cycles

// Pipelined execution

FOR each clock cycle

Perform IF for next instruction

Perform ID for previous instruction

Perform EX for previous instruction

Perform MEM for previous instruction

Perform WB for previous instruction

END FOR

Calculate pipeline_cycles

Speedup =

non_pipeline_cycles / pipeline_cycles

Throughput =

number_of_instructions / pipeline_cycles

Display results

STOP

16. Observation Table

| MeasurementResult |
| --- |
| Number of pipeline stages | 5 |
| Number of instructions | 5 |
| Non-pipelined cycles | 25 |
| Pipelined cycles | 9 |
| Speedup | 2.78× |
| Average throughput | 0.556 instructions/cycle |
| Ideal maximum speedup | Approximately 5× |

17. Analysis of Results

The experiment demonstrates that pipelining significantly improves processor performance by allowing different instructions to be processed simultaneously.

In the non-pipelined system, only one instruction is being processed at a time. Therefore, the processor requires 25 cycles for five instructions under the simplified model.

In the pipelined system, the first instruction requires five cycles to pass through all stages. After the pipeline becomes full, subsequent instructions progress through different stages simultaneously. Consequently, only nine cycles are required for the five instructions.

The resulting speedup is:

925​≈2.78

The experiment therefore validates the main advantage of instruction pipelining: higher instruction throughput through overlapping execution.

However, the speedup does not reach the theoretical maximum of 5× because the pipeline requires startup cycles. In real processors, hazards and unequal stage delays can further reduce the achievable performance.

18. Advantages of Pipelining

- Increases instruction throughput.

- Allows overlapping instruction execution.

- Improves utilization of processor components.

- Reduces the average time between completed instructions.

- Provides a foundation for advanced processor architectures.

19. Limitations

- Pipeline hazards can reduce performance.

- Additional hardware is required.

- Control logic becomes more complex.

- Pipeline stalls can reduce throughput.

- Branch instructions can introduce control penalties.

- Unequal stage delays can limit the clock frequency.

20. Result

The five-stage instruction pipeline was successfully simulated using the stages Fetch, Decode, Execute, Memory and Write-back. For five independent instructions, the non-pipelined system required 25 cycles, whereas the pipelined system required 9 cycles.

The calculated speedup was approximately:

2.78×​

The experiment demonstrates that pipelining improves processor performance by allowing multiple instructions to execute concurrently.

21. Conclusion

The experiment successfully demonstrates the effectiveness of a 5-stage instruction pipeline. By dividing instruction execution into IF, ID, EX, MEM and WB stages, multiple instructions can occupy different stages simultaneously.

Compared with non-pipelined execution, the pipelined processor achieved significantly lower total execution time and higher instruction throughput. Although ideal pipelining can approach a speedup equal to the number of stages, practical performance is affected by startup overhead and pipeline hazards.

Thus, instruction pipelining is a fundamental technique for improving processor throughput and forms an important foundation for modern high-performance processors. This directly fulfills the assessment requirement to measure speedup and throughput and compare pipelined execution with a non-pipelined system.

## 1. Aim

To develop a pipeline execution scenario in which **data hazards, control hazards, and structural hazards** occur, observe their impact on instruction execution, and implement **stalling, data forwarding, and branch prediction** techniques to resolve the hazards and improve processor performance. This is the fourth experiment specified in the assessment. 

## 2. Objectives

- To understand different types of pipeline hazards. 
- To identify data, control and structural hazards. 
- To simulate the effect of hazards on instruction execution. 
- To implement pipeline stalling. 
- To implement data forwarding. 
- To understand branch prediction. 
- To compare pipeline performance before and after hazard resolution. 

---

# 3. Introduction

A pipeline allows several instructions to execute simultaneously in different stages. However, overlapping instructions can create situations where an instruction cannot safely proceed.

These situations are called **pipeline hazards**.

The three major types are:

1. **Data Hazard** 
2. **Control Hazard** 
3. **Structural Hazard** 

Hazards can cause:

- Pipeline stalls 
- Delayed execution 
- Reduced throughput 
- Increased execution time 

The purpose of hazard-handling techniques is to maintain correct execution while minimizing performance loss.

---

# 4. Types of Pipeline Hazards

## A. Data Hazard

A data hazard occurs when an instruction depends on the result produced by an earlier instruction that has not yet completed.

Example:

```
I1: ADD R1, R2, R3
```

I2: SUB R4, R1, R5

Here, I2 requires the value of **R1**, which is produced by I1.

Therefore:

```
I1 → produces R1
```

↓

I2 → requires R1

This creates a dependency.

### RAW Hazard

The above example is a **Read After Write (RAW)** hazard.

```
I1: Write R1
```

I2: Read R1

---

# 5. Data Hazard Without Forwarding

Consider:

```
I1: ADD R1, R2, R3
```

I2: SUB R4, R1, R5

Pipeline execution:

| CycleI1I2 | | |
| --------- | --- | --- |
| 1 | IF | |
| 2 | ID | IF |
| 3 | EX | ID |
| 4 | MEM | EX |
| 5 | WB | MEM |
| 6 | | WB |

At the EX stage of I2, the required R1 value may not yet be available from the normal register-write path.

Therefore, a **stall** may be required depending on the datapath design.

---

# 6. Stalling

A **stall** temporarily stops an instruction from progressing through the pipeline.

A bubble is inserted into the pipeline.

Conceptually:

```
Instruction → Wait → Continue
```

Example:

| CycleI1I2I3 | | | |
| ----------- | --- | ----- | --- |
| 1 | IF | | |
| 2 | ID | IF | |
| 3 | EX | ID | IF |
| 4 | MEM | STALL | ID |
| 5 | WB | EX | EX |
| 6 | | MEM | MEM |
| 7 | | WB | WB |

The stall prevents I2 from using an incorrect or unavailable value.

### Advantage

- Maintains correctness. 
- Simple to implement. 

### Disadvantage

- Increases execution time. 
- Reduces pipeline throughput. 

---

# 7. Data Forwarding

Instead of waiting for a result to be written back to the register file, the result can be directly forwarded from a later pipeline stage to an earlier stage.

Example:

```
I1: ADD R1, R2, R3
```

I2: SUB R4, R1, R5

Without forwarding:

```
I1 → WB → R1 → I2
```

With forwarding:

```
┌─────────────┐
```

I1 ALU result│ ▼

─────────────┴────────► I2 ALU

The result produced by I1 can be forwarded directly to the ALU input required by I2.

### Forwarding Path

```
┌──────────┐
```

R2 ─────────►│ │

R3 ─────────►│ ALU │

└────┬─────┘

│

│ Result

▼

Forwarding Unit

│

▼

Next ALU input

Forwarding can eliminate many unnecessary stalls.

---

# 8. Control Hazard

A control hazard occurs when the processor encounters a **branch or jump instruction** and does not yet know which instruction should be fetched next.

Example:

```
I1: BEQ R1, R2, TARGET
```

I2: ADD R3, R4, R5

I3: SUB R6, R7, R8

If the branch is taken, I2 and I3 may be instructions from the wrong path.

Therefore, the processor must determine the branch outcome before continuing with the correct instruction stream.

---

# 9. Control Hazard Without Branch Prediction

The processor can wait until the branch condition is resolved.

```
Branch
```

↓

Wait for decision

↓

Determine target

↓

Fetch correct instruction

This creates pipeline bubbles and reduces performance.

---

# 10. Branch Prediction

**Branch prediction** attempts to predict whether a branch will be taken before the final branch decision is available.

Two basic approaches are:

### Static Prediction

The processor uses a fixed rule.

Examples:

```
Always Taken
```

Always Not Taken

### Dynamic Prediction

The processor uses information from previous branch behavior.

A simple dynamic predictor can use a **1-bit prediction state**:

```
Taken → predict Taken
```

Not Taken → predict Not Taken

The prediction is updated when the actual branch result becomes known.

---

# 11. Structural Hazard

A structural hazard occurs when two instructions require the **same hardware resource at the same time**.

Example:

Suppose the processor has a single memory unit used for both:

- Instruction fetching 
- Data access 

Consider:

```
I1: LOAD R1, [100]
```

I2: ADD R2, R3, R4

If I1 requires memory access in the MEM stage while another instruction simultaneously requires instruction memory access, both cannot use the same resource at the same time.

---

# 12. Structural Hazard Resolution

A structural hazard can be handled by:

### Method 1 – Stalling

One instruction waits until the resource becomes available.

### Method 2 – Additional Hardware

Separate resources can be provided.

For example:

```
Instruction Memory ───► IF
```

│

▼

Data Memory ───────────► MEM

Using separate instruction and data memories reduces resource conflicts.

---

# 13. Combined Hazard Scenario

A more realistic experiment can combine all three hazards.

Consider:

```
I1: LOAD R1, [R2]
```

I2: ADD R3, R1, R4

I3: BEQ R3, R5, TARGET

I4: SUB R6, R7, R8

I5: OR R9, R10, R11

Here:

### Data Hazard

```
I1 → produces R1
```

I2 → requires R1

### Control Hazard

```
I3 → branch instruction
```

### Structural Hazard

Can be introduced by configuring the simulation with a shared memory resource required by two pipeline stages simultaneously.

---

# 14. Pipeline Without Hazard Handling

A simplified execution sequence may look like:

```
Cycle → 1 2 3 4 5 6 7 8
```



I1 IF ID EX MEM WB

I2 IF ID EX MEM WB

I3 IF ID EX MEM WB

I4 IF ID EX MEM WB

I5 IF ID EX MEM WB

However, this ideal schedule does not account for the dependencies and branch behavior.

Consequently, incorrect data or incorrect instruction execution could occur if hazards are ignored.

---

# 15. Pipeline With Hazard Resolution

After introducing forwarding, stalls and branch prediction:

```
Cycle → 1 2 3 4 5 6 7 8 9
```



I1 IF ID EX MEM WB

I2 IF ID EX MEM WB

↑

Forwarding



I3 IF ID EX MEM WB

↑

Branch prediction

Structural conflicts are handled by either scheduling/stalling or providing separate resources.

---

# 16. Hazard Resolution Table

| HazardCauseEffectResolution | | | |
| --------------------------- | -------------------------------------- | ------------------------------------ | ------------------------------ |
| Data | Instruction depends on previous result | Incorrect/late operand | Forwarding / Stalling |
| Control | Branch changes instruction flow | Wrong instructions fetched | Branch prediction / Flush |
| Structural | Hardware resource conflict | Instructions cannot proceed together | Stalling / Additional hardware |

---

# 17. Experimental Procedure

1. Design a 5-stage pipeline. 
2. Define a set of dependent instructions. 
3. Introduce a data dependency. 
4. Observe the resulting data hazard. 
5. Execute the instructions without forwarding. 
6. Record the stalls. 
7. Enable data forwarding. 
8. Compare the execution schedule. 
9. Introduce a branch instruction. 
10. Observe the control hazard. 
11. Implement branch prediction. 
12. Introduce a shared hardware resource to create a structural hazard. 
13. Resolve it using stalling or additional hardware. 
14. Compare execution time before and after hazard handling. 

---

# 18. Pseudocode

```
START
```



Initialize five pipeline stages:

IF, ID, EX, MEM, WB



Load instructions



Check for DATA hazards



IF dependency exists THEN

IF forwarding is available THEN

Forward required result

ELSE

Insert pipeline stall

END IF

END IF



Check for CONTROL hazards



IF branch instruction exists THEN

Predict branch direction



IF prediction is correct THEN

Continue execution

ELSE

Flush incorrect instructions

Fetch correct target

END IF

END IF



Check for STRUCTURAL hazards



IF two instructions require same resource THEN

Stall one instruction

OR provide an additional resource

END IF



Execute remaining instructions



Measure total cycles



STOP

---

# 19. Observation Table

| ConfigurationHazard HandlingExpected Effect | | |
| ------------------------------------------- | ------------------- | ----------------------------------- |
| Basic pipeline | None | Hazard-related delays/errors |
| Pipeline + Stall | Stalling | Correct execution, lower throughput |
| Pipeline + Forwarding | Data forwarding | Fewer data stalls |
| Pipeline + Branch Prediction | Prediction | Reduced branch penalty |
| Pipeline + Resource Duplication | Additional hardware | Reduced structural conflicts |

---

# 20. Performance Analysis

The major objective is not simply to remove hazards, but to **minimize their performance penalty while maintaining correct execution**.

### Without forwarding

A dependent instruction may need to wait for the previous instruction to complete.

```
More stalls
```

↓

More cycles

↓

Lower throughput

### With forwarding

The result can be supplied directly to the dependent instruction.

```
Forward result
```

↓

Reduce waiting

↓

Fewer stalls

↓

Higher throughput

### With branch prediction

The processor can continue fetching instructions based on the predicted branch direction.

A correct prediction reduces waiting.

A wrong prediction causes the incorrectly fetched instructions to be discarded, creating a **branch penalty**.

---

# 21. Comparative Analysis

| TechniqueMain PurposePerformance Effect | | |
| --------------------------------------- | ------------------------------ | --------------------------------- |
| Stalling | Prevent incorrect execution | Correct but slower |
| Data Forwarding | Reduce data dependency delay | Improves throughput |
| Branch Prediction | Reduce branch waiting | Improves control-flow performance |
| Resource Duplication | Avoid structural conflicts | Enables more parallel execution |
| Pipeline Flush | Remove wrong-path instructions | Necessary after misprediction |

---

# 22. Advantages of Hazard Handling

- Maintains correct instruction execution. 
- Reduces unnecessary pipeline stalls. 
- Improves instruction throughput. 
- Improves utilization of processor resources. 
- Allows deeper and more efficient pipelines. 
- Reduces the performance impact of dependencies and branches. 

---

# 23. Limitations

- Forwarding requires additional hardware and control logic. 
- Branch prediction can be incorrect. 
- Incorrect predictions cause pipeline flushing. 
- Stalls still consume clock cycles. 
- Additional hardware increases processor complexity. 
- Structural hazard prevention can increase hardware cost. 

---

# 24. Result

The pipeline hazard experiment successfully demonstrates **data, control and structural hazards** and their effects on instruction execution.

The experiment also demonstrates three important resolution approaches:

- **Stalling** for preventing unsafe execution. 
- **Data forwarding** for reducing data dependency delays. 
- **Branch prediction** for reducing control-hazard penalties. 

These techniques improve pipeline efficiency while preserving correct instruction execution. The experiment directly satisfies the assessment requirement to observe hazards and implement stalling, forwarding and branch prediction. 

---

# 25. Conclusion


The experiment demonstrates that appropriate hazard-resolution mechanisms can significantly improve pipeline performance. **Data forwarding reduces unnecessary waiting, stalling ensures correct execution when dependencies cannot be resolved immediately, and branch prediction reduces the penalty associated with control flow changes.

Therefore, effective hazard management is essential for designing a high-performance pipelined processor.




 1. Aim

To analyze a processor supporting **superscalar execution**, including **out-of-order execution and speculative execution**, and to extend the study to **Hyper-Threading and Simultaneous Multithreading (SMT)**. The performance improvement is evaluated in terms of **instruction throughput and resource utilization**. This corresponds directly to Experiment 5 in the assessment. 

## 2. Objectives

- To understand superscalar processor organization. 
- To study parallel execution of multiple instructions. 
- To understand out-of-order execution. 
- To understand speculative execution. 
- To study Hyper-Threading and SMT. 
- To compare instruction throughput. 
- To analyze processor resource utilization. 

---

# 3. Introduction

A traditional scalar processor generally issues and executes approximately **one instruction at a time**.

A **superscalar processor**, however, contains multiple execution resources and can issue multiple independent instructions during the same clock cycle.

Conceptually:

```
Instruction Stream
```

│

▼

┌─────────────┐

│ Instruction │

│ Fetch │

└──────┬──────┘

│

▼

┌─────────────┐

│ Decode │

└──────┬──────┘

│

┌────────────┼────────────┐

▼ ▼ ▼

Integer Floating Memory

Unit Unit Unit

│ │ │

└────────────┼─────────────┘

▼

Results

The objective is to exploit **instruction-level parallelism (ILP)**.

---

# 4. Superscalar Execution

A superscalar processor can issue multiple instructions in one clock cycle when suitable hardware resources and independent instructions are available.

For example, consider:

```
I1: ADD R1, R2, R3
```

I2: SUB R4, R5, R6

I3: AND R7, R8, R9

I4: OR R10, R11, R12

If two execution units are available, two independent instructions can potentially execute simultaneously.

```
Cycle 1:
```

I1 → ALU 1

I2 → ALU 2



Cycle 2:

I3 → ALU 1

I4 → ALU 2

Instead of processing one instruction at a time, the processor can exploit parallelism.

---

# 5. Superscalar Architecture

A simplified architecture is:

```
┌─────────────┐
```

│ Instruction │

│ Fetch │

└──────┬──────┘

▼

┌─────────────┐

│ Decode │

└──────┬──────┘

▼

┌─────────────┐

│ Issue Unit │

└──────┬──────┘

│

┌──────────┼──────────┐

▼ ▼ ▼

ALU 1 ALU 2 Load/Store

│ │ │

└──────────┼──────────┘

▼

Write-back

The **issue unit** determines which instructions can execute in parallel.

---

# 6. Out-of-Order Execution

In a simple in-order processor, instructions are executed according to their original program order.

In **out-of-order execution**, an instruction can execute before an earlier instruction if:

- Its operands are available. 
- The required execution unit is available. 
- Executing it will not violate program correctness. 

### Example

Consider:

```
I1: LOAD R1, [100]
```

I2: ADD R4, R5, R6

I3: SUB R7, R8, R9

Suppose I1 is delayed because of memory access.

I2 and I3 do not depend on R1.

Therefore:

```
I1 → Waiting for memory
```

I2 → Execute

I3 → Execute

This prevents the processor from remaining idle while an independent instruction is ready.

---

# 7. In-Order vs Out-of-Order

| FeatureIn-OrderOut-of-Order | | |
| --------------------------- | ------------- | --------------------------------- |
| Execution order | Program order | Availability based |
| Parallelism | Lower | Higher |
| Hardware complexity | Lower | Higher |
| Resource utilization | Lower | Better |
| Handling delays | Often waits | Executes independent instructions |
| Performance | Lower | Higher |

---

# 8. Speculative Execution

**Speculative execution** allows the processor to execute instructions before it is completely certain that those instructions belong to the correct execution path.

It is particularly useful for branch instructions.

Example:

```
Branch
```

│

┌────┴────┐

▼ ▼

Path A Path B

The processor predicts which path is likely to be executed and begins processing instructions from that path.

If the prediction is correct:

```
Prediction correct
```

↓

Continue execution

↓

Performance improves

If the prediction is incorrect:

```
Prediction incorrect
```

↓

Discard speculative work

↓

Fetch correct instructions

Thus, speculative execution can improve performance but requires mechanisms to recover from incorrect predictions.

---

# 9. Relationship Between Prediction and Speculation

The two concepts work together:

```
Branch Instruction
```

↓

Branch Prediction

↓

Predict Target Path

↓

Speculatively Execute

↓

Branch Outcome Known

↓

┌──────────────┐

│ │

▼ ▼

Correct Incorrect

│ │

▼ ▼

Commit Discard/Recover

---

# 10. Reorder Buffer Concept

Out-of-order and speculative processors require mechanisms to maintain correct program behavior.

A simplified **Reorder Buffer (ROB)** can be used to keep track of instructions.

```
Instruction
```

↓

Reorder Buffer

↓

Execute when ready

↓

Wait for completion

↓

Commit in program order

This allows instructions to execute out of order while their architectural results are committed in the correct order.

---

# 11. Hyper-Threading and SMT

**Simultaneous Multithreading (SMT)** allows a processor core to manage instructions from multiple threads at the same time.

The term **Hyper-Threading** is a technology name used by Intel for its implementation of simultaneous multithreading.

The main idea is:

```
Physical Core
```

│

┌────────┴────────┐

▼ ▼

Thread 1 Thread 2

│ │

└────────┬────────┘

▼

Shared Resources

If Thread 1 cannot fully utilize a processor resource, Thread 2 can potentially use the available capacity.

---

# 12. SMT Example

Suppose a processor has two logical threads:

```
Thread 1:
```

I1 I2 I3



Thread 2:

J1 J2 J3

The processor may schedule instructions from both threads:

| CycleThread 1Thread 2 | | |
| --------------------- | -- | -- |
| 1 | I1 | J1 |
| 2 | I2 | J2 |
| 3 | I3 | J3 |

The exact execution depends on instruction dependencies and available processor resources.

---

# 13. Resource Utilization

A major benefit of SMT is improved utilization of processor resources.

Without SMT:

```
Thread 1
```

↓

Resource utilization

↓

Some resources may remain idle

With SMT:

```
Thread 1 ──┐
```

├──► Shared execution resources

Thread 2 ──┘

The second thread can use resources that might otherwise remain unused.

---

# 14. Experimental Scenario

Consider a processor with:

- 2 integer execution units 
- 1 memory unit 
- 1 floating-point unit 
- 2 hardware threads 

Instructions:

```
Thread 1:
```

I1: ADD

I2: SUB

I3: LOAD



Thread 2:

J1: MUL

J2: ADD

J3: LOAD

The processor attempts to schedule independent instructions according to resource availability.

---

# 15. Execution Without Superscalar Processing

Assume one instruction is issued per cycle.

For six instructions:

```
I1
```

I2

I3

J1

J2

J3

Idealized issue time:

6 cycles 

Therefore:

Throughput=66​=1 

So the simplified throughput is:

1 instruction/cycle​ 

---

# 16. Superscalar Execution

If two independent instructions can be issued per cycle:

```
Cycle 1 → I1 + J1
```

Cycle 2 → I2 + J2

Cycle 3 → I3 + J3

Idealized issue time:

3 cycles 

Throughput:

36​=2 

Therefore:

2 instructions/cycle​ 

The idealized issue throughput is doubled.

---

# 17. Speedup

Using the simplified scenario:

Speedup=36​ 2×​ 

Thus, a two-wide superscalar processor can theoretically provide up to **2× issue throughput** for a workload containing enough independent instructions.

Actual speedup is usually lower because of:

- Dependencies 
- Branches 
- Memory delays 
- Resource conflicts 
- Limited instruction-level parallelism 

---

# 18. Comparison of Processor Techniques

| FeatureScalarSuperscalarOut-of-OrderSpeculativeSMT | | | | | |
| -------------------------------------------------- | -------- | --------------- | ------ | --------- | ------------------ |
| Multiple instructions/cycle | Limited | Yes | Yes | Yes | Yes |
| Instruction parallelism | Low | High | Higher | Higher | Higher |
| Executes out of order | No | Not necessarily | Yes | Often | Can |
| Uses branch prediction | Possible | Common | Common | Essential | Common |
| Multiple threads | No | No | No | No | Yes |
| Resource utilization | Lower | Higher | Higher | Higher | Potentially higher |
| Complexity | Low | Medium | High | High | High |

---

# 19. Experimental Procedure

1. Design a basic scalar processor model. 
2. Define a set of independent instructions. 
3. Measure the instruction issue rate. 
4. Extend the processor to support multiple execution units. 
5. Implement superscalar instruction issue. 
6. Introduce an instruction dependency. 
7. Implement out-of-order scheduling. 
8. Introduce a branch instruction. 
9. Implement speculative execution using branch prediction. 
10. Introduce two instruction streams. 
11. Implement SMT-style resource sharing. 
12. Measure instruction throughput. 
13. Compare resource utilization between configurations. 

---

# 20. Pseudocode

```
START
```



Initialize processor resources



Create instruction stream



// Scalar execution

FOR each instruction

Issue one instruction

Execute instruction

END FOR



Measure scalar throughput



// Superscalar execution

WHILE instructions remain

Identify independent instructions

Issue multiple instructions

Execute using available functional units

END WHILE



Measure superscalar throughput



// Out-of-order execution

Identify ready instructions

Execute ready instructions first

Maintain correct commit order



// Speculative execution

Predict branch

Execute predicted path



IF prediction is correct

Commit results

ELSE

Discard speculative results

Recover correct path

END IF



// SMT

Create Thread 1 and Thread 2

Share processor execution resources

Schedule ready instructions from both threads



Measure final throughput and resource utilization



STOP

---

# 21. Observation Table

| Processor ConfigurationIdealized Issue RateRelative Throughput | | |
| -------------------------------------------------------------- | ------------------------------ | --------------------------- |
| Scalar | 1 instruction/cycle | 1× |
| 2-wide Superscalar | 2 instructions/cycle | 2× |
| Out-of-order | Depends on available ILP | Higher potential |
| Speculative execution | Depends on prediction accuracy | Higher potential |
| SMT | Depends on thread workload | Better resource utilization |

---

# 22. Analysis

The experiment demonstrates that modern processors improve performance by exploiting different forms of parallelism.

### Superscalar Execution

Allows multiple independent instructions to be issued during the same cycle.

### Out-of-Order Execution

Prevents delays in one instruction from unnecessarily stopping independent instructions.

### Speculative Execution

Allows the processor to continue working along a predicted control-flow path instead of waiting for every branch decision.

### SMT

Allows instructions from multiple threads to share execution resources and can improve utilization when one thread cannot fully occupy the available resources.

These techniques complement each other:

```
Superscalar
```

↓

Multiple instructions

↓

Out-of-order scheduling

↓

Use ready instructions

↓

Speculative execution

↓

Reduce branch waiting

↓

SMT

↓

Use resources across threads

↓

Higher potential throughput

---

 23. Performance Factors

The theoretical throughput does not always translate directly into real-world speedup.

Performance can be limited by:

- Data dependencies 
- Control dependencies 
- Memory latency 
- Branch mispredictions 
- Execution-unit availability 
- Cache misses 
- Insufficient instruction-level parallelism 
- Competition between threads 

Therefore, increasing the number of execution units alone does not guarantee proportional performance improvement.

---

 24. Advantages

### Superscalar

- Executes multiple instructions simultaneously. 
- Increases instruction throughput. 
- Improves utilization of execution units. 

### Out-of-Order Execution

- Reduces waiting caused by slow instructions. 
- Exploits independent instructions. 
- Improves execution-unit utilization. 

### Speculative Execution

- Reduces the effect of branch delays. 
- Keeps execution units busy. 
- Improves performance when predictions are accurate. 

 SMT

- Allows multiple threads to share a physical processor core. 
- Can improve resource utilization. 
- Can increase throughput for suitable workloads. 



25. Limitations

- Significant hardware complexity. 
- Increased power consumption. 
- More complex scheduling mechanisms. 
- Speculative execution can waste work after incorrect predictions. 
- SMT threads compete for shared resources. 
- Performance depends strongly on workload characteristics. 


 26. Result

The experiment analyzed a processor using **superscalar execution, out-of-order execution, speculative execution and SMT**.

In the simplified two-wide superscalar example, six instructions could theoretically be issued in three cycles instead of six cycles, giving:

2× idealized issue speedup​ 

The analysis also demonstrated that out-of-order execution can exploit independent instructions, speculative execution can reduce branch-related delays, and SMT can improve utilization by allowing instructions from multiple threads to share processor resources.



27. Conclusion

The experiment demonstrates how modern processor organizations exploit **instruction-level and thread-level parallelism** to improve performance.

Superscalar execution allows multiple instructions to be processed in parallel, while out-of-order execution ensures that ready instructions can proceed without unnecessarily waiting for delayed instructions. Speculative execution reduces the performance cost of control-flow decisions, and SMT allows multiple threads to utilize the resources of a processor core.