SIMATS ENGINEERING

Department of Computer Science & Engineering

ASSESSMENT TOOL 1- INDUSTRY PROBLEM SOLVING TASK

| Course Code: | CSA12 | Course Title: | Computer Architecture |
| --- | --- | --- | --- |
| CO Assessed: | CO4 | Assessment: | ASSESSMENT TOOL1- INDUSTRY PROBLEM SOLVING TASK |
| Weightage: | 30% | Total Marks: | 25 |
| CO4 Statement: | Implement hierarchical memory systems by differentiating cache levels (L1, L2, L3), virtual memory with paging, and advanced memory technologies (DRAM, SRAM, NAND Flash, MRAM, RRAM) based on latency, bandwidth, and throughput characteristics. |  |  |
| Student Name: | Shaswath A S | Reg.No.: | 192511195 |
| Department: | CSE | Date: | 21.08.2026 |

ANNEXURE A

INDUSTRY PROBLEM SOLVING TASK

- Smartphone Performance Optimization                                 
 A mobile manufacturer observes lag while switching between apps. Analyze the role of L1, L2, L3 cache and DRAM and propose a hierarchical memory redesign to reduce latency and improve throughput.

- Cloud Server Memory Bottleneck
 A cloud service provider experiences high latency in database queries. Compare cache hierarchy vs virtual memory paging and recommend improvements using appropriate memory technologies.

- Autonomous Vehicle Data Processing
 An autonomous vehicle must process sensor data in real time. Evaluate SRAM, DRAM, and MRAM and design a memory hierarchy that maximizes bandwidth and minimizes latency.

- AI Inference Edge Device
 An edge AI device needs fast model loading with low power consumption. Compare NAND Flash, DRAM, and RRAM and recommend an optimized hierarchical memory structure.

- High-Performance Gaming Console
 A gaming console suffers frame drops during large scene loading. Analyze L3 cache vs DRAM throughput and propose memory hierarchy enhancements.

Code of Conduct:

I Shaswath A S / 192511195 (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

MARKS ALLOCATION

|  | Understanding of Industry Problem (20%) | Application of Engineering Concepts (25%) | Solution Design (25%) | Use of Tools (15%) | Analysis (10%) |
| --- | --- | --- | --- | --- | --- |
| Q1 |  |  |  |  |  |
| Q2 |  |  |  |  |  |
| Q3 |  |  |  |  |  |
| Q4 |  |  |  |  |  |
| Q5 |  |  |  |  |  |

RUBRICS FOR CALCULATION:

| Criteria | Weightage | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Understanding of Industry Problem | 20% | Comprehensive understanding of real-world problem and constraints | Good understanding with minor gaps in requirements | Basic understanding; some requirements unclear | Poor understanding; misinterprets problem context |
| Application of Engineering Concepts | 25% | Applies advanced and relevant concepts effectively to solve the problem | Applies appropriate concepts with minor errors | Limited application of concepts | Incorrect or no application of concepts |
| Solution Design | 25% | Innovative, practical, and feasible solution aligned with industry standards | Logical and feasible solution with minor gaps | Basic solution with limited feasibility | Unclear or impractical solution |
| Use of Tools | 15% | Effective use of modern tools, technologies, and real data | Appropriate use of tools with correct implementation | Limited or inconsistent use of tools | Minimal or incorrect use of tools |
| Analysis | 10% | Thorough analysis with validated results and performance evaluation | Good analysis with reasonable validation | Basic analysis with limited validation | Poor or no analysis |

Q1. Smartphone Performance Optimization

Problem Understanding

A smartphone manufacturer observes noticeable lag when users switch between applications. The problem is primarily associated with the time required to access application instructions and data from memory. Modern smartphones execute many applications concurrently, so frequently accessed data must be kept close to the processor to minimize memory-access latency.

The memory system should therefore provide:

- Very low latency for frequently accessed data.

- Sufficient capacity for application working sets.

- High bandwidth for simultaneous CPU/GPU/AI workloads.

- Efficient movement of data between cache levels and DRAM.

- Low power consumption because smartphones operate under strict energy constraints.

Engineering Analysis

The processor uses a hierarchical memory system:

| Memory | Typical Role | Latency | Capacity | Main Advantage |
| --- | --- | --- | --- | --- |
| L1 Cache | Immediate CPU data/instructions | Lowest | Very small | Extremely fast |
| L2 Cache | Secondary CPU cache | Very low | Small | Reduces L1 misses |
| L3 Cache | Shared/last-level cache | Low | Larger | Reduces DRAM accesses |
| DRAM | Main memory | Higher | Large | High capacity and bandwidth |

L1 cache should contain the most frequently accessed instructions and data. It provides the lowest latency but has very limited capacity.

L2 cache acts as the next level when data is not found in L1. It provides a balance between speed and capacity.

L3 cache is larger and can be shared among processor cores. It reduces the number of expensive DRAM accesses, particularly when several applications or cores access related data.

DRAM provides the main working memory for applications. It has substantially greater capacity than cache but higher access latency.

The key principle is:

L1 → L2 → L3 → DRAM

As we move down the hierarchy, capacity increases while access speed generally decreases.

Proposed Hierarchical Memory Redesign

The smartphone should adopt a multi-level cache and DRAM architecture:

CPU Core

│

├── L1 Instruction Cache

├── L1 Data Cache

│

└── L2 Cache

│

└── Shared L3 Cache

│

└── High-bandwidth Mobile DRAM

Frequently used application data should remain in L1/L2 whenever possible. Data shared across cores should benefit from L3. DRAM should hold the larger application working sets.

The operating system can additionally improve responsiveness through:

- Intelligent application-memory prioritization.

- Cache-aware scheduling.

- Efficient memory compression where appropriate.

- Prefetching for predictable workloads.

- Avoiding unnecessary application eviction and reloads.

- Optimizing background-process memory usage.

Use of Tools and Technologies

Performance can be evaluated using processor performance-monitoring counters and operating-system profiling tools. Measurements should include:

- L1 cache hit/miss rate.

- L2 cache hit/miss rate.

- L3 cache hit/miss rate.

- DRAM bandwidth.

- Memory stalls.

- Application-switch latency.

- CPU utilization.

A profiling workflow can compare the existing and proposed architectures under identical workloads.

Analysis and Validation

The redesign should be considered successful if application-switch latency decreases while cache hit rates and effective memory bandwidth increase.

A useful performance metric is:

Average Memory Access Time (AMAT)

AMAT can be conceptually represented as:

AMAT = Hit Time + Miss Rate × Miss Penalty

Increasing the hit rate of L1/L2/L3 reduces the number of accesses reaching DRAM and therefore reduces the effective memory-access time.

Final Recommendation

The manufacturer should prioritize a balanced L1–L2–L3 cache hierarchy combined with high-bandwidth mobile DRAM rather than simply increasing DRAM capacity. The most important improvement for application-switching responsiveness is keeping frequently reused data close to the processor.

Expected result: lower latency, fewer processor stalls, improved application-switch responsiveness, and better overall throughput without excessive power consumption.

Q2. Cloud Server Memory Bottleneck

Problem Understanding

The cloud provider experiences high database-query latency. Database applications repeatedly access indexes, records, buffers, and frequently used tables. If these data structures are not available in cache or physical memory, the processor may experience expensive memory accesses and, in severe cases, virtual-memory paging.

The problem requires distinguishing between cache hierarchy and virtual memory paging.

Cache Hierarchy vs Virtual Memory Paging

| Feature | Cache Hierarchy | Virtual Memory Paging |
| --- | --- | --- |
| Main purpose | Reduce CPU memory-access latency | Extend apparent memory capacity |
| Managed mainly by | Hardware/cache controller | OS + hardware MMU |
| Unit | Cache line | Page |
| Speed | Very high | Much slower when storage is involved |
| Best for | Frequently reused data | Memory capacity/isolation |
| Main problem when ineffective | Cache misses | Page faults |

Cache hierarchy improves performance by keeping frequently accessed data close to the CPU.

Virtual memory allows processes to use an address space larger than available physical RAM. However, when a required page is absent from physical memory, a page fault can occur. If the page must be retrieved from storage, latency becomes extremely high compared with DRAM access.

Therefore, virtual memory should not be treated as a replacement for adequate DRAM.

Proposed Cloud Memory Architecture

CPU

│

├── L1 Cache

│

├── L2 Cache

│

└── L3 Cache

│

└── Large-Capacity DRAM

│

└── NVMe/SSD

│

└── Persistent Storage

The database buffer pool should be designed so that frequently accessed database pages remain in DRAM.

The system should prioritize:

- Increasing effective cache hit rates.

- Providing sufficient DRAM for the database working set.

- Reducing unnecessary page faults.

- Using fast storage for unavoidable paging and persistent data.

- Optimizing database indexes and query access patterns.

Engineering Improvements

1. Increase DRAM Capacity

If the database working set is larger than available RAM, frequently accessed pages may be repeatedly evicted. Increasing DRAM capacity can reduce this problem.

2. Improve Cache Locality

Database structures should be organized to improve spatial and temporal locality. Frequently accessed indexes and data should remain memory-resident.

3. Reduce Paging

The operating system should avoid excessive swapping. Memory allocation should leave sufficient RAM for the database buffer pool and other essential services.

4. Use High-Performance Storage

Where storage access is unavoidable, NVMe-based storage can reduce I/O latency compared with slower storage technologies.

Tools and Technologies

The cloud provider can use:

- Linux memory-monitoring tools.

- Database performance-monitoring tools.

- CPU performance counters.

- Database query profilers.

- I/O monitoring.

- Cache and page-fault statistics.

Important measurements include:

- Query response time.

- Cache hit ratio.

- DRAM utilization.

- Page-fault rate.

- Storage I/O latency.

- Memory bandwidth.

- CPU stall cycles.

Analysis and Validation

The system should first establish a baseline query latency. After increasing DRAM and optimizing cache usage, the same database workload should be executed again.

A successful implementation should show:

- Lower query latency.

- Higher database buffer-cache hit ratio.

- Fewer page faults.

- Reduced storage I/O.

- Better CPU utilization.

- Higher transaction throughput.

Final Recommendation

The provider should optimize cache hierarchy and provide sufficient high-bandwidth DRAM before relying on virtual-memory paging. Virtual memory is primarily a capacity and isolation mechanism, whereas cache and DRAM directly address performance.

The recommended architecture is therefore:

L1/L2/L3 Cache → High-capacity DRAM → NVMe Storage

This provides a practical balance between latency, capacity, throughput, and cost.

Q3. Autonomous Vehicle Data Processing

Problem Understanding

An autonomous vehicle must process data from cameras, radar, LiDAR, ultrasonic sensors, GPS, and other systems in real time. Delayed processing can affect perception, decision-making, and vehicle control.

Therefore, the memory system must provide:

- Extremely low latency.

- Very high bandwidth.

- Deterministic and predictable access.

- High reliability.

- Efficient operation under continuous workloads.

Comparison of SRAM, DRAM, and MRAM

| Feature | SRAM | DRAM | MRAM |
| --- | --- | --- | --- |
| Volatile | Yes | Yes | No |
| Speed | Extremely high | High | High |
| Density | Low | High | Medium |
| Power for refresh | No refresh | Requires refresh | No DRAM-style refresh |
| Typical role | CPU/cache memory | Main memory | Fast persistent/specialized memory |
| Main limitation | Low density/high cost | Higher latency and refresh | Cost/density trade-offs |

SRAM

SRAM is ideal for very latency-sensitive data because it provides extremely fast access. Its major limitation is low density and relatively high cost per bit. It is therefore appropriate for CPU caches and small high-speed buffers.

DRAM

DRAM offers much greater density and is suitable for storing large sensor-processing datasets, intermediate results, and application working sets.

MRAM

MRAM is non-volatile and can provide fast access while retaining data without continuous power. It can be useful for selected persistent or low-latency storage requirements, configuration data, and specialized memory applications.

Proposed Memory Hierarchy

Sensor Processing Accelerators / CPU

│

Small SRAM Buffers

│

L1 / L2 / L3 SRAM Cache

│

High-Bandwidth DRAM

│

MRAM / Persistent

Fast Data and Parameters

SRAM Layer

Sensor data requiring immediate processing should be placed in SRAM/cache. Examples include:

- Current control-loop data.

- Frequently accessed sensor metadata.

- Critical intermediate computation values.

DRAM Layer

Large streams of camera, LiDAR, and radar data should be stored in high-bandwidth DRAM.

MRAM Layer

MRAM can store persistent configuration and selected data that benefits from non-volatility and relatively fast access.

Engineering Optimization

The vehicle should use DMA (Direct Memory Access) and dedicated accelerators where appropriate so that large sensor streams can move between devices and memory without unnecessarily occupying CPU resources.

Memory buffers should be carefully organized to maximize:

- Sequential access.

- Parallel access.

- Cache locality.

- Memory-channel utilization.

- Data reuse.

Double-buffering or multi-buffering can allow one sensor-data buffer to be processed while another is being filled.

Tools and Technologies

The system can be evaluated using:

- Hardware performance counters.

- Real-time operating-system profiling.

- Memory-bandwidth benchmarks.

- Sensor-data replay systems.

- FPGA/accelerator profiling tools.

- CPU/GPU/NPU utilization monitoring.

Important validation parameters include:

Sensor-to-decision latency, memory bandwidth, cache-miss rate, buffer utilization, and processing throughput.

Analysis and Validation

Suppose sensor data arrives continuously at a high data rate. The architecture must ensure that the memory subsystem can sustain the incoming data rate while simultaneously providing data to perception and control algorithms.

A useful condition is:

Available memory bandwidth ≥ Required sensor-data bandwidth + Processing traffic

If this condition is not satisfied, buffers will accumulate data and processing latency will increase.

Final Recommendation

A three-level hierarchy using SRAM/cache for critical low-latency data, high-bandwidth DRAM for large sensor workloads, and MRAM for selected persistent/fast-access data provides the best balance.

The design maximizes bandwidth by using DRAM for bulk data while minimizing latency by keeping critical data in SRAM/cache.

Q4. AI Inference Edge Device

Problem Understanding

An edge AI device must load machine-learning models quickly while consuming limited power. Examples include smart cameras, industrial sensors, robotics, and portable AI systems.

The memory hierarchy must balance:

- Model-loading speed.

- Inference latency.

- Memory capacity.

- Power consumption.

- Non-volatility.

- Cost.

Comparison of NAND Flash, DRAM, and RRAM

| Technology | Volatile? | Speed | Capacity | Main Role |
| --- | --- | --- | --- | --- |
| NAND Flash | No | Lower than DRAM | High | Permanent model storage |
| DRAM | Yes | High | Medium/High | Active model execution |
| RRAM | No | Potentially faster than conventional storage | Application dependent | Emerging fast non-volatile memory |

NAND Flash

NAND Flash is suitable for permanent storage of AI models because it is non-volatile and offers high density. However, it is significantly slower than working memory.

DRAM

DRAM is suitable for holding the active neural-network model, intermediate tensors, and runtime data. It provides substantially faster access than NAND Flash.

RRAM

RRAM is an emerging non-volatile memory technology that can potentially combine persistence with relatively fast access and low-energy operation. It is particularly interesting for edge-AI architectures.

Proposed Hierarchical Structure

AI Accelerator / CPU / NPU

│

On-chip SRAM

│

DRAM

│

RRAM

│

NAND Flash

Level 1 – SRAM

Frequently reused weights, activations, and intermediate values should be placed in on-chip SRAM where possible.

Level 2 – DRAM

The active model and larger intermediate tensors should be maintained in DRAM.

Level 3 – RRAM

RRAM can serve as a fast non-volatile model-storage layer in architectures where the technology is economically and technically suitable.

Level 4 – NAND Flash

NAND Flash provides high-capacity persistent storage for complete models, firmware, and less frequently accessed data.

Power Optimization

The system should avoid repeatedly loading the complete model from NAND Flash. Instead:

- Load the required model into DRAM.

- Keep frequently accessed portions in SRAM.

- Reuse model data as much as possible.

- Reduce unnecessary DRAM transfers.

- Use model compression/quantization where application accuracy permits.

- Power-gate or place unused memory components into low-power states.

Tools and Technologies

Evaluation can use:

- AI inference benchmarking.

- Memory-bandwidth profiling.

- Power/energy monitoring.

- NPU/GPU profiling.

- Model-loading benchmarks.

- Cache and SRAM utilization analysis.

Important metrics include:

- Model-loading time.

- Inference latency.

- Inferences per second.

- Energy per inference.

- DRAM bandwidth.

- Memory utilization.

Analysis and Validation

The proposed design should be compared against a conventional architecture in which the model is repeatedly loaded from NAND Flash.

Expected improvements include:

- Reduced model-loading latency.

- Lower storage traffic.

- Lower energy consumption.

- Higher inference throughput.

- Improved responsiveness.

Final Recommendation

The optimal structure is:

SRAM → DRAM → RRAM → NAND Flash

SRAM provides the fastest working memory, DRAM supports active inference workloads, RRAM provides an opportunity for fast persistent storage, and NAND Flash provides economical high-capacity storage.

This hierarchy balances latency, bandwidth, capacity, persistence, and power consumption.

Q5. High-Performance Gaming Console

Problem Understanding

A gaming console experiences frame drops while loading large scenes. Modern games continuously stream textures, geometry, shaders, audio, and other assets. If the memory subsystem cannot supply these assets quickly enough, the GPU may wait for data, causing frame-time spikes and visible frame drops.

The key issue is the relationship between L3 cache capacity/latency and DRAM bandwidth.

L3 Cache vs DRAM

| Feature | L3 Cache | DRAM |
| --- | --- | --- |
| Latency | Lower | Higher |
| Capacity | Smaller | Much larger |
| Bandwidth | High for cached data | High bulk bandwidth |
| Cost per bit | Higher | Lower |
| Best use | Frequently reused data | Large working datasets |

L3 cache is useful for frequently accessed data because it is much closer to the processor than DRAM.

However, a large game scene can contain data far beyond the capacity of L3. Therefore, simply increasing L3 cache will not solve the entire problem.

DRAM provides the capacity and bandwidth required for large game assets.

Proposed Memory Architecture

CPU/GPU

│

L1 Cache

│

L2 Cache

│

Shared L3 Cache

│

High-Bandwidth DRAM

│

Fast Persistent Storage

The console should use L3 for frequently reused CPU/game-engine data while relying on high-bandwidth DRAM for large scene data.

Design Improvements

1. Increase Effective L3 Utilization

Frequently reused game-engine data, object metadata, physics information, and frequently accessed instructions should benefit from L3.

2. Increase DRAM Bandwidth

Large textures, geometry, and rendering resources should be transferred through high-bandwidth memory channels.

3. Improve Asset Streaming

The storage subsystem should stream assets ahead of their use rather than waiting until the GPU requires them.

4. Use Prefetching

Predictable scene transitions can be used to prefetch assets into DRAM before they are needed.

5. Avoid Memory Contention

CPU, GPU, and other accelerators should be scheduled so that one workload does not unnecessarily starve another of memory bandwidth.

Use of Tools

The console manufacturer can use:

- GPU profiling tools.

- CPU performance counters.

- Frame-time analysis.

- Memory-bandwidth monitoring.

- Asset-streaming profilers.

- Cache-miss measurements.

The most important metrics are:

- Average frame time.

- 1% low frame rate.

- Frame-time spikes.

- L3 hit rate.

- DRAM bandwidth utilization.

- Asset-loading latency.

- GPU idle/wait time.

Analysis and Validation

For a 60 FPS target, the theoretical frame budget is approximately:

Frame time = 1 / 60 ≈ 16.67 ms

If scene loading causes individual frames to exceed this budget significantly, the player may perceive stuttering or frame drops.

The redesigned architecture should therefore minimize situations where the GPU waits for memory or asset transfers.

A controlled test should compare:

Existing architecture vs. redesigned cache/DRAM/streaming architecture

under identical large-scene workloads.

Success should be demonstrated by:

- Reduced frame-time spikes.

- Higher sustained frame rates.

- Improved DRAM utilization.

- Reduced CPU/GPU waiting.

- Improved asset-streaming performance.

Final Recommendation

The console should not rely solely on increasing L3 cache. Instead, it should combine:

Optimized L1/L2/L3 cache hierarchy + high-bandwidth DRAM + intelligent asset streaming/prefetching.

L3 should retain frequently reused game-engine data, while DRAM should provide the bandwidth and capacity required for large scenes.