SIMATS ENGINEERING

Department of Computer Science & Engineering

ASSESSMENT TOOL 2- COMPARATIVE ANALYSIS

| Course Code: | CSA12 | Course Title: | Computer Architecture |
| --- | --- | --- | --- |
| CO Assessed: | CO4 | Assessment: | ASSESSMENT TOOL2- COMPARATIVE ANALYSIS |
| Weightage: | 30% | Total Marks: | 25 |
| CO4 Statement: | Implement hierarchical memory systems by differentiating cache levels (L1, L2, L3), virtual memory with paging, and advanced memory technologies (DRAM, SRAM, NAND Flash, MRAM, RRAM) based on latency, bandwidth, and throughput characteristics. |  |  |
| Student Name: | Shaswath A S | Reg.No.: | 192511195 |
| Department: | CSE0 | Date: | 22 08 2026 |

ANNEXURE A

COMPARATIVE ANALYSIS

- Compare L1, L2, and L3 cache levels in terms of latency, bandwidth, and throughput, and analyze their impact on processor performance.

- Differentiate SRAM and DRAM by evaluating their latency, bandwidth, and throughput characteristics in cache and main memory design.

- Compare virtual memory with paging and cache memory mechanisms in terms of access latency and system throughput.

- Analyze the differences between NAND Flash and DRAM with respect to latency, bandwidth, and suitability for hierarchical memory systems.

- Compare MRAM and RRAM technologies by examining their performance in terms of access time, throughput, and energy efficiency.

Code of Conduct:

I Shaswath A S / 192511195  (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

MARKS ALLOCATION

|  | Scope of Items (20%) | Comparison Depth (25%) | Use of Evidence (20%) | Critical Thinking (20%) | Clarity of Presentation (15%) |
| --- | --- | --- | --- | --- | --- |
| Q1 |  |  |  |  |  |
| Q2 |  |  |  |  |  |
| Q3 |  |  |  |  |  |
| Q4 |  |  |  |  |  |
| Q5 |  |  |  |  |  |

RUBRICS FOR CALCULATION:

| Criteria | Weightage | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Scope of Items | 20% | Selects highly relevant items with clear scope and justification | Selects appropriate items with minor gaps | Selection is limited or partially relevant | Items are unclear or not relevant |
| Comparison Depth | 25% | Provides detailed and comprehensive comparison across multiple aspects | Provides adequate comparison with some depth | Limited comparison with few aspects | Very minimal or no comparison |
| Use of Evidence | 20% | Supports comparison with accurate data, examples, or references | Uses some supporting evidence with minor gaps | Limited or weak supporting evidence | No supporting evidence provided |
| Critical Thinking | 20% | Demonstrates strong critical thinking with clear interpretation and reasoning | Shows reasonable interpretation with minor gaps | Basic interpretation; lacks depth | No meaningful interpretation |
| Clarity of Presentation | 15% | Well-organized, clear, and logically structured presentation | Generally clear with minor issues | Somewhat unclear or poorly structured | Disorganized and difficult to understand |

Answers :

Q1. Compare L1, L2, and L3 Cache Levels in Terms of Latency, Bandwidth, and Throughput, and Analyze Their Impact on Processor Performance

Introduction

Cache memory is a small, high-speed memory located close to or inside the processor. Its main purpose is to reduce the time required by the CPU to access frequently used instructions and data. Modern processors generally use a hierarchical cache system consisting of L1, L2, and L3 caches. Each level provides a trade-off between speed, capacity, bandwidth, and cost.

1. L1 Cache

L1 cache is the fastest and smallest cache level. It is located closest to the CPU execution units and is normally divided into:

- Instruction cache (L1-I) – stores frequently used instructions.

- Data cache (L1-D) – stores frequently accessed data.

Because L1 has very low latency, the processor can retrieve required information extremely quickly. However, its small capacity means that it cannot store a large amount of data.

Impact on performance:
A high L1 cache hit rate allows the CPU to obtain data without accessing slower cache levels or RAM. Therefore, L1 cache has a direct and significant effect on CPU performance.

2. L2 Cache

L2 cache is larger than L1 but slightly slower. It acts as a secondary cache when the required information is not available in L1.

For example:

CPU → L1 → L2 → L3 → Main Memory

If the processor experiences an L1 cache miss but finds the required data in L2, it avoids the much larger delay associated with accessing main memory.

Impact on performance:
L2 improves performance by providing a relatively fast backup for L1 and reducing the number of accesses to slower memory.

3. L3 Cache

L3 cache is generally larger but slower than L1 and L2. In many modern processors, L3 is shared among several CPU cores.

Its larger capacity allows it to hold more data and instructions. It is especially useful when multiple cores need access to common information.

Impact on performance:
L3 reduces expensive main-memory accesses and helps maintain performance when the working set of an application is too large for L1 and L2.

Latency, Bandwidth and Throughput Analysis

Latency refers to the time required to access data. L1 has the lowest latency, followed by L2 and L3.

Bandwidth refers to the amount of data that can be transferred per unit of time. L1 generally provides the highest effective bandwidth because of its proximity to the CPU and its small size.

Throughput represents how much useful processing or data transfer can be completed over time. Higher cache hit rates generally improve processor throughput because the CPU spends less time waiting for data.

The relationship can be represented as:

L1 → Lowest latency / Highest effective bandwidth
L2 → Moderate latency / High bandwidth
L3 → Higher latency / Large capacity
RAM → Much higher latency

Critical Analysis

The three cache levels work together rather than competing with each other. L1 provides maximum speed, L2 provides additional fast storage, and L3 provides larger shared capacity.

A processor with a very small L1 cache may experience more L1 misses, increasing dependence on L2 and L3. On the other hand, increasing cache size does not automatically make a processor faster because larger caches can require more access time and consume more chip area and power.

Therefore, an effective cache hierarchy balances latency, capacity, bandwidth, power consumption, and cost.

Example

Consider a program repeatedly processing the same set of data. If the required data is found in L1, the CPU can access it very quickly. If it is not in L1 but is present in L2, the CPU experiences a slightly greater delay. If both L1 and L2 miss but L3 contains the data, the delay increases further. If all cache levels miss, the processor must access main memory, resulting in substantially higher latency.

Conclusion

L1, L2, and L3 caches form an essential hierarchical memory system that improves processor performance. L1 offers the fastest access, L2 provides additional low-latency storage, and L3 provides larger shared capacity. By increasing cache hit rates and reducing accesses to main memory, the hierarchy improves processor throughput and overall execution performance.

Q2. Differentiate SRAM and DRAM by Evaluating Their Latency, Bandwidth, and Throughput Characteristics in Cache and Main Memory Design

Introduction

SRAM (Static Random Access Memory) and DRAM (Dynamic Random Access Memory) are two important types of semiconductor memory used in computer systems. They differ mainly in their speed, storage density, cost, power requirements, and applications. SRAM is commonly used for cache memory, while DRAM is primarily used as main memory (RAM).

1. SRAM in Cache Memory

SRAM is preferred for cache memory because it provides very fast access with low latency. It does not require periodic refresh operations to maintain stored data.

The CPU frequently accesses cache memory during instruction execution. Therefore, the low latency of SRAM allows the processor to retrieve frequently used data and instructions quickly.

Advantages of SRAM for cache:

- Very low access latency

- High-speed operation

- Suitable for frequent CPU accesses

- No periodic refresh requirement

- Supports high processor performance

However, SRAM requires more hardware per stored bit, making it larger and more expensive than DRAM. This limits the amount of SRAM that can economically be placed in a processor.

2. DRAM in Main Memory

DRAM is widely used as the main memory of computers because it provides much greater storage density at a lower cost per bit.

Unlike SRAM, DRAM cells require periodic refreshing to maintain their stored information. This introduces additional memory-management activity and contributes to its higher effective access latency.

Advantages of DRAM for main memory:

- High storage density

- Lower cost per bit

- Suitable for large memory capacities

- Efficient for storing large amounts of program and data information

Although DRAM has higher latency than SRAM, its large capacity makes it practical for main memory.

Latency Comparison

Latency is the time taken to access required data.

SRAM has lower latency because its storage mechanism allows fast direct access without the refresh process associated with DRAM.

Therefore:

SRAM → Lower latency → Suitable for CPU cache

DRAM → Higher latency → Suitable for main memory

The processor first attempts to obtain frequently required information from cache. If the information is not available there, it accesses main memory, which generally takes longer.

Bandwidth and Throughput

Bandwidth refers to the amount of data that can be transferred within a given period, while throughput refers to the amount of useful data or processing completed over time.

SRAM provides excellent performance for the small, frequent accesses made by processors. DRAM, because of its larger capacity and memory-interface design, is well suited to transferring larger quantities of data between main memory and the processor.

Thus, the two memories are optimized for different purposes:

- SRAM: optimized primarily for very fast, low-latency access.

- DRAM: optimized primarily for high-capacity and economical data storage with substantial memory bandwidth.

Cache and Main Memory Design

The difference between SRAM and DRAM explains their positions in the memory hierarchy:

CPU → SRAM Cache → DRAM Main Memory → Secondary Storage

SRAM is placed close to the CPU because the processor requires extremely fast access to frequently used information. DRAM is placed below cache because the system requires much greater capacity than can practically be provided using SRAM.

Critical Analysis

Choosing between SRAM and DRAM is not simply a question of which memory is faster. A computer architecture must balance latency, capacity, cost, density, power, bandwidth, and throughput.

If SRAM were used for all main memory, the system could benefit from lower latency, but the cost and physical area required would become impractical. Conversely, using only DRAM for cache would reduce cost and increase capacity, but the processor would experience higher access latency.

Therefore, using SRAM for cache and DRAM for main memory provides an effective compromise between speed and capacity.

Example

When a processor repeatedly accesses the same variable during program execution, storing that information in SRAM-based cache allows rapid access. If the required information is not available in cache, the processor retrieves it from DRAM. The additional latency of DRAM makes cache hits important for maintaining high processor performance.

Conclusion

SRAM and DRAM serve complementary roles in computer memory architecture. SRAM offers lower latency and fast access, making it ideal for CPU cache, whereas DRAM offers greater density and lower cost per bit, making it suitable for main memory. Their combination creates an efficient memory hierarchy that balances performance, capacity, bandwidth, throughput, and cost.

This directly addresses the assessment requirement to compare SRAM and DRAM in terms of latency, bandwidth, throughput, and their roles in cache and main-memory design.

Q3. Compare Virtual Memory with Paging and Cache Memory Mechanisms in Terms of Access Latency and System Throughput

Introduction

Virtual memory, paging, and cache memory are important mechanisms used to improve the efficiency of a computer's memory system. Although both paging and cache mechanisms help manage data access, they operate at different levels and solve different problems. Cache memory primarily reduces CPU access latency, whereas virtual memory and paging allow programs to use an address space larger than the available physical RAM.

1. Cache Memory Mechanism

Cache memory stores frequently or recently accessed instructions and data close to the processor.

When the CPU requests data:

CPU → Cache → Main Memory

If the required data is found in the cache, it is called a cache hit. The processor can access it with very low latency.

If the data is not found, a cache miss occurs, and the system must retrieve the required information from a lower level of the memory hierarchy.

Impact on Latency

Cache memory significantly reduces access latency because it is much faster and closer to the processor than main memory.

Impact on Throughput

A high cache-hit rate reduces CPU waiting time. Consequently, the processor can execute more instructions within a given period, improving overall system throughput.

2. Virtual Memory

Virtual memory allows a program to operate with a logical address space that can be larger than the available physical RAM.

The operating system uses secondary storage as an extension of physical memory when necessary.

A simplified representation is:

CPU → Virtual Address → Memory Management Unit → Physical Memory

If the required data is available in RAM, it can be accessed normally. If it is not present, the operating system may need to retrieve it from secondary storage.

Impact on Latency

Virtual memory can have significantly higher latency when the required information is not present in physical RAM. Accessing secondary storage is much slower than accessing RAM.

Impact on Throughput

Virtual memory allows larger applications and multiple processes to operate even when their total logical memory requirements exceed available RAM. However, excessive movement of pages between RAM and storage can reduce system throughput.

3. Paging

Paging is a memory-management technique used to implement virtual memory. The virtual address space is divided into fixed-size pages, while physical memory is divided into corresponding frames.

The operating system maps virtual pages to physical frames.

For example:

Virtual Page → Physical Frame

If the required page is already present in RAM, the processor can continue execution. If it is absent, a page fault occurs, and the operating system must bring the page into physical memory.

Impact on Latency

A normal page access that finds the page in RAM has relatively manageable latency. However, a page fault can introduce a very large delay because secondary storage must be accessed.

Impact on Throughput

Paging improves memory utilization and enables multiple programs to share physical memory efficiently. However, frequent page faults can cause substantial performance degradation.

Access Latency Comparison

The general relationship can be represented as:

Cache hit < RAM access < Storage access caused by page fault

This means cache memory is optimized for minimum access latency, while virtual memory and paging prioritize memory capacity and process management.

Throughput Comparison

Cache memory generally improves throughput by reducing the time the CPU spends waiting for memory.

Virtual memory and paging can improve overall system utilization by allowing more programs to coexist in memory. However, excessive page faults can reduce throughput significantly.

Therefore:

- Cache → Improves CPU performance and throughput

- Virtual memory → Increases usable memory capacity

- Paging → Provides an organized mechanism for managing virtual memory

Critical Analysis

Cache memory and virtual memory should not be considered alternatives. They operate at different levels of the memory hierarchy.

Cache memory attempts to keep frequently accessed data close to the CPU, while paging determines which parts of a program should reside in physical memory.

A system can therefore use both mechanisms simultaneously:

CPU → Cache → RAM → Secondary Storage

The effectiveness of the complete system depends on maintaining a high cache-hit rate and avoiding excessive page faults.

Example

Suppose a program repeatedly accesses the same data.

- If the data is present in cache, the CPU gets it quickly.

- If it is not in cache but is available in RAM, the system accesses RAM.

- If the required page is not in RAM, a page fault occurs and the operating system must retrieve it from secondary storage.

Thus, the location of data directly affects access latency and system throughput.

Conclusion

Cache memory, virtual memory, and paging perform different but complementary functions. Cache memory primarily reduces CPU access latency and improves processor throughput, while virtual memory provides a larger logical memory space and paging manages that space by dividing memory into pages and frames. Although virtual memory and paging improve memory utilization, frequent page faults can greatly increase latency and decrease throughput. Therefore, an efficient memory hierarchy requires both fast cache access and effective virtual-memory management.

Q4. Analyze the Differences Between NAND Flash and DRAM with Respect to Latency, Bandwidth, and Suitability for Hierarchical Memory Systems

Introduction

NAND Flash and DRAM are both semiconductor memory technologies, but they are designed for different purposes. DRAM is primarily used as main memory because it provides relatively fast access and high bandwidth, while NAND Flash is commonly used for non-volatile storage such as SSDs and memory cards.

1. DRAM

DRAM is widely used as the main memory of a computer. It provides relatively low access latency compared with storage technologies and supports frequent read and write operations required during program execution.

The CPU uses DRAM when required data is not available in cache.

A simplified hierarchy is:

CPU → Cache → DRAM → NAND Flash Storage

Advantages of DRAM

- Low access latency

- High memory bandwidth

- Suitable for frequent random accesses

- Supports active program execution

- Faster read/write operations than NAND Flash

Limitation

DRAM is volatile, meaning stored information is lost when power is removed. It also requires periodic refresh operations.

2. NAND Flash

NAND Flash is a non-volatile memory technology. It retains data even when the system is powered off.

It is commonly used in:

- Solid-state drives (SSDs)

- USB flash drives

- Memory cards

- Embedded storage

Unlike DRAM, NAND Flash is not normally used as the primary working memory of a processor because its access latency is considerably higher.

Latency

Latency is one of the most important differences between the two technologies.

DRAM has much lower access latency than NAND Flash.

DRAM can therefore respond efficiently to the processor's frequent memory requests. NAND Flash is better suited to persistent storage where capacity and data retention are more important than extremely low latency.

Bandwidth

Bandwidth represents the amount of data that can be transferred over a period of time.

DRAM provides high bandwidth for continuous interaction with the CPU and is therefore suitable for main-memory workloads.

Modern NAND-based storage devices can also provide substantial sequential data-transfer bandwidth, particularly when multiple NAND channels operate in parallel. However, high sequential bandwidth does not mean NAND Flash has DRAM-like access latency.

This distinction is important:

High bandwidth does not necessarily mean low latency.

Therefore, NAND Flash can provide impressive bulk-transfer performance while still being unsuitable as a direct replacement for DRAM in conventional main-memory systems.

Suitability in Hierarchical Memory Systems

DRAM and NAND Flash occupy different positions in the memory hierarchy.

Typical hierarchy:

CPU Registers → L1/L2/L3 Cache → DRAM → NAND Flash Storage

DRAM's Role

DRAM acts as the working memory for active programs and data. Its relatively low latency allows the processor to access information frequently during execution.

NAND Flash's Role

NAND Flash acts as persistent storage. It provides much larger capacity and retains information without power.

When a program is loaded from NAND Flash, the required information is transferred into DRAM before the CPU works extensively with it.

Critical Analysis

Replacing DRAM completely with NAND Flash would provide greater storage capacity and non-volatility, but it would significantly increase memory-access latency and negatively affect processor performance.

Similarly, replacing NAND Flash with DRAM would provide faster access but would be impractical for large-scale persistent storage because DRAM is volatile and generally more expensive per bit.

Therefore, the two technologies are complementary rather than interchangeable.

The architecture uses each technology where its characteristics are most beneficial:

- DRAM → speed and active processing

- NAND Flash → capacity and persistent storage

Example

Consider opening a large application.

- The application is permanently stored on an SSD using NAND Flash.

- When the application starts, required program data is loaded from NAND Flash into DRAM.

- Frequently accessed information is then brought into CPU cache.

- The CPU primarily works with the cache and DRAM during execution.

This demonstrates how NAND Flash and DRAM work together within the hierarchical memory system.

Conclusion

DRAM and NAND Flash have fundamentally different characteristics. DRAM provides lower latency and high bandwidth, making it suitable for main memory, whereas NAND Flash provides high density, non-volatility, and economical persistent storage. Although NAND Flash can provide high sequential bandwidth, its access latency is much higher than DRAM. Therefore, an efficient hierarchical memory system uses DRAM for active processing and NAND Flash for long-term storage.

Q5. Compare MRAM and RRAM Technologies by Examining Their Performance in Terms of Access Time, Throughput, and Energy Efficiency

Introduction

MRAM (Magnetoresistive Random Access Memory) and RRAM (Resistive Random Access Memory) are emerging non-volatile memory technologies designed to overcome some limitations of conventional memory technologies. Both retain data without continuous power and have potential applications in future memory hierarchies.

1. MRAM

MRAM stores information using magnetic states rather than electrical charge. Because the stored information is magnetic, MRAM can retain data even when power is removed.

One of its major advantages is its combination of fast access and non-volatility.

Access Time

MRAM can provide relatively fast read and write access compared with many conventional non-volatile memory technologies.

This makes it attractive for applications where the system requires both speed and data persistence.

Throughput

MRAM can support high-performance memory operations. Its fast access characteristics allow processors and embedded systems to handle frequent memory operations efficiently.

Energy Efficiency

Since MRAM is non-volatile, it does not need continuous power to maintain stored information. This can reduce standby power consumption and make MRAM attractive for energy-conscious systems.

2. RRAM

RRAM stores information by changing the resistance of a material between different states.

For example, a memory cell may represent different logical states depending on whether its resistance is relatively high or low.

RRAM is attractive because of its:

- High potential storage density

- Non-volatility

- Low-energy operation potential

- Fast switching capability

- Potential scalability

Access Time

RRAM can provide fast switching and access, although actual performance depends strongly on the specific RRAM material, cell structure, and implementation.

Throughput

RRAM has the potential to provide high throughput because its cells can be switched between resistance states rapidly. This makes it a promising technology for high-density memory and specialized computing architectures.

Energy Efficiency

RRAM can potentially operate with low energy per memory operation. Its non-volatile nature also reduces the need for continuous power to preserve stored information.

Access Time Comparison

Both technologies are designed to provide fast access compared with many traditional non-volatile storage technologies.

MRAM:
Fast access with mature high-speed memory applications.

RRAM:
Potentially very fast switching, but performance varies according to the underlying device technology.

Therefore, neither technology can be declared universally superior without considering the specific implementation and workload.

Throughput Comparison

Throughput depends on several factors, including:

- Read/write speed

- Memory architecture

- Number of memory cells accessed simultaneously

- Controller design

- Parallelism

MRAM provides high-performance operation suitable for embedded and memory applications. RRAM also has strong potential for high throughput, particularly when implemented with high-density arrays and parallel architectures.

Thus, both MRAM and RRAM can support high-throughput applications, but the actual result depends on the architecture in which the technology is implemented.

Energy Efficiency Comparison

Energy efficiency is an important advantage of emerging non-volatile memories.

MRAM does not require periodic refreshing to preserve stored information. This can reduce standby energy compared with volatile memories.

RRAM also has the potential for low-energy operation because information is stored through resistance states rather than requiring continuous charge maintenance.

Therefore:

MRAM → Strong combination of speed, endurance and non-volatility

RRAM → Strong potential for low energy, high density and scalability

Critical Analysis

MRAM and RRAM should not be viewed simply as replacements for each other. Their suitability depends on the requirements of the application.

MRAM is attractive when:

- Fast access is required

- High endurance is important

- Non-volatility is required

- Reliable embedded memory is needed

RRAM is attractive when:

- High density is important

- Low energy operation is desired

- Scalable memory arrays are required

- Emerging computing architectures are being considered

RRAM also has potential beyond conventional memory, including applications where the resistance states can be used for computational purposes.

Example

In an embedded system, MRAM could store important configuration information that must remain available after power is removed while still requiring fast access during operation.

RRAM could be used in a high-density memory architecture where low energy consumption and scalability are important design goals.

Conclusion

MRAM and RRAM are promising non-volatile memory technologies with the potential to improve future memory systems. MRAM offers a strong combination of fast access, high endurance, and non-volatility, while RRAM provides promising characteristics in switching speed, energy efficiency, density, and scalability. The appropriate technology depends on the application's requirements for access time, throughput, endurance, density, and energy consumption.