SIMATS ENGINEERING

Department of Computer Science & Engineering

CO5 – ASSESSMENT TOOL 2
PSEUDOCODE WRITING (ALGORITHM DESIGN)

| Student Name | Shaswath A S |
| --- | --- |
| Register Number | 192511195 |

Question 1 – DMA-Based Data Transfer

ALGORITHM DMA_DATA_TRANSFER(Device, Memory, Size)
BEGIN
    Configure DMA with source = Device and destination = Memory
    Set DMA transfer_count = Size
    Enable DMA completion interrupt
    Start DMA
    WHILE DMA.status != COMPLETE DO
        CPU executes other tasks
        IF DMA.status = ERROR THEN
            Stop DMA
            Report error
            RETURN FAILURE
        END IF
    END WHILE
    ON DMA_INTERRUPT
        Acknowledge interrupt
        Verify transferred_count = Size
        IF verification successful THEN
            Mark transfer as COMPLETE
        ELSE
            Mark transfer as FAILED
        END IF
    END INTERRUPT
    RETURN SUCCESS
END

Question 2 – Prioritized Vectored Interrupts

ALGORITHM HANDLE_VECTORED_INTERRUPTS()
BEGIN
    LOOP FOREVER
        IF Interrupt_Pending() THEN
            vector ← Get_Highest_Priority_Vector()
            Save_Minimum_Context()
            SWITCH vector
                CASE TIMER_VECTOR: Timer_ISR()
                CASE STORAGE_VECTOR: Storage_ISR()
                CASE NETWORK_VECTOR: Network_ISR()
                DEFAULT: Generic_ISR()
            END SWITCH
            Acknowledge_Interrupt(vector)
            Restore_Context()
        END IF
    END LOOP
END

Question 3 – Dynamic I/O Mapping

ALGORITHM SELECT_IO_MAPPING(latency_requirement, bandwidth_requirement)
BEGIN
    IF latency_requirement = VERY_LOW THEN
        mode ← MEMORY_MAPPED_IO
    ELSE IF bandwidth_requirement = HIGH THEN
        mode ← MEMORY_MAPPED_IO
    ELSE IF address_space_is_limited = TRUE THEN
        mode ← IO_MAPPED_IO
    ELSE
        mode ← MEMORY_MAPPED_IO
    END IF
    Configure_Device(mode)
    RETURN mode
END

Question 4 – Bus Arbitration

ALGORITHM BUS_ARBITRATION(requests)
BEGIN
    Initialize queue for PCIe, USB and Thunderbolt
    Initialize wait_time for every device = 0
    LOOP WHILE requests are pending
        FOR each requesting device DO
            wait_time[device] ← wait_time[device] + 1
        END FOR
        eligible ← devices with pending requests
        selected ← Select_Highest_Priority(eligible)
        IF multiple devices have same priority THEN
            selected ← Round_Robin_Select(eligible)
        END IF
        Grant_Bus(selected)
        Transfer_Data(selected, selected.interface)
        Release_Bus(selected)
        wait_time[selected] ← 0
    END LOOP
END

Question 5 – Hybrid I/O Communication

ALGORITHM HYBRID_IO_COMMUNICATION(device)
BEGIN
    speed ← Get_Device_Speed(device)
    IF speed = LOW THEN
        REPEAT
            status ← Poll_Device_Status(device)
            IF status = DATA_READY THEN
                data ← Read_Device_Data(device)
                Process(data)
            END IF
            Wait(short_interval)
        UNTIL Device_Disconnected(device)
    ELSE
        Configure_DMA(device)
        Enable_DMA_Completion_Interrupt(device)
        Start_DMA(device)
        ON DMA_COMPLETION_INTERRUPT
            Acknowledge_Interrupt()
            Process_Completed_Buffer()
            IF More_Data_Available(device) THEN
                Restart_DMA(device)
            END IF
        END INTERRUPT
    END IF
END

Student Declaration

I, Shaswath A S (Reg No: 192511195), certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.